RISC style CPU16 with Custom ISA:

ISA:

| Opcode (hex) | Mnemonic | Effect |
| --- | --- | --- |
| 0x0 | NOP | EAT 5* |
| 0x1 | MOV | R[rd]<- {8’b0,imm8} |
| 0x2 | ADD | R[rd] ← R[rs1] + R[rs2} |
| 0x3 | SUB | R[rd] ← R[rs1] - R[rs2] |
| 0x4 | AND | R[rd] ← R[rs1] & R[rs2] |
| 0x5 | OR | R[rd] ← R[rs1] \¦ R[rs2] |
| 0x6 | XOR | R[rd] ← R[rs1] ^ R[rs2] |
| 0x7 | MUL | R[rd] ← R[rs1] * R[rs2] |
| 0x8 | MAC | R[1] ← R[1] + (R[rs1] * R[rs2]) |
| 0x9 | NN | R[rd] ← (R[rs1]·ws + R[rs2]·w2 + bias)[15:0] |
| 0Xa | MATMUL | R[rd] ← matmul_part ( c00) |
| 0xb | MOV | R[rd] ← R[rs1] |
| 0xc | OUT | trace output |
| 0Xd | NOT | ~INPUT |
| 0xf | STOP | $finish |

DEMO PROGRAMS:

First program

LOADI R1, 10

LOADI R2, 5

ADD R3, R1, R2

SUB R4, R2, R4

AND R5, R2, R4

OR R6, R2, R4

XOR R7, R3, R2

MUL R5, R2, R4

SIMD R3, R2, R4

MAC R4, R2, R4

NN R1

MATMUL R0

STOP

Full coverage test case

1001  // LOADI R0 1

1102  // LOADI R1 2

1203  // LOADI R2 3

1704  // LOADI R7 4

2301  // ADD  R3 R0 R1

4412  // AND  R4 R1 R2

5512  // OR   R5 R1 R2

6612  // XOR  R6 R1 R2

7712  // MUL  R7 R1 R2

9012  // NN   R0        (uses R1, R2, R7)

A145  // MATMUL R1      (uses R0–R7 for A, B matrices)

B65   // MOV  R6 R5

E000  // OUT  R0

E110  // OUT  R1

F000  // STOP

All mod covered

LOADI R0 1      // ALU base: R0 = 1

LOADI R1 2      // R1 = 2

LOADI R2 3      // R2 = 3

LOADI R3 4      // R3 = 4

ADD  R4 R0 R1   // ALU ADD: 1+2 = 3

SUB  R5 R2 R1   // ALU SUB: 3–2 = 1

AND  R6 R2 R3   // ALU AND: 3 & 4

OR   R7 R2 R3   // ALU OR:  3 | 4

XOR  R0 R2 R3   // ALU XOR: 3 ^ 4

MUL  R1 R2 R3   // ALU MUL: 3 × 4 = 12

MAC  R2 R4 R5   // MAC: R2 ← R2 + R4×R5    (core MAC module)

MAC2 R3 R6 R7   // MAC2: R3 ← R3 + R6×R7  (second MAC path, if you kept 0xC as MAC2)

MOV  R4 R1      // MOV:  R4 ← R1

NN   R0         // NN: R0 ← NN(x1=R1, x2=R2, b=R7)

// effectively uses MAC‑like sums (x1·w1 + x2·w2 + b)

MATMUL R5       // MATMUL: R5 ← c00 (A×B)

// where A = [R0,R1; R2,R3], B = [R4,R5; R6,R7]

OUT  R0         // output R0 (NN result)

OUT  R1         // output R1

OUT  R2         // output R2 (MAC result)

OUT  R3         // output R3 (MAC2 result)

OUT  R5         // output R5 (MATMUL result)

STOP

ASSEMBLER.C: MNEMONICS -> HEX

#include <stdio.h>

#include <stdlib.h>

#include <string.h>

#include <ctype.h>

int reg_num(const char *r)

{

if (r == NULL || toupper((unsigned char)r[0]) != 'R')

return -1;

int n = atoi(r + 1);

if (n < 0 || n > 7)

return -1;

return n;

}

void trim_comment(char *line)

{

for (int i = 0; line[i]; i++)

{

if (line[i] == ';' || line[i] == '#')

{

line[i] = '\0';

return;

}

}

}

int main(void)

{

FILE *fin = fopen("E:\\vhdl programs\\CPU16modular\\CPU16modular.srcs\\sim_1\\new\\input.txt", "r");

FILE *fout = fopen("E:\\vhdl programs\\CPU16modular\\CPU16modular.srcs\\sim_1\\new\\program.txt", "w");

if (!fin)

{

printf("Cannot open input.txt\n");

return 1;

}

if (!fout)

{

printf("Cannot create program.txt\n");

fclose(fin);

return 1;

}

char line[128];

int line_no = 0;

while (fgets(line, sizeof(line), fin))

{

line_no++;

trim_comment(line);

char op[20] = {0}, a[20] = {0}, b[20] = {0}, c[20] = {0};

unsigned short instr = 0;

int count = sscanf(line, "%19s %19s %19s %19s", op, a, b, c);

if (count <= 0)

continue;

for (int i = 0; op[i]; i++)

op[i] = toupper((unsigned char)op[i]);

if (strcmp(op, "NOP") == 0)

{

instr = 0x0000;

}

else if (strcmp(op, "STOP") == 0)

{

instr = 0xF000;

}

else if (strcmp(op, "LOADI") == 0)

{

int rd = reg_num(a);

int imm = atoi(b);

if (rd < 0)

{

printf("Line %d: invalid register in LOADI\n", line_no);

continue;

}

instr = (0x1 << 12) | (rd << 9) | (imm & 0xFF);

}

else if (strcmp(op, "ADD") == 0 || strcmp(op, "SUB") == 0 ||

strcmp(op, "AND") == 0 || strcmp(op, "OR") == 0 ||

strcmp(op, "XOR") == 0 || strcmp(op, "MUL") == 0 ||

strcmp(op, "MAC") == 0)

{

int rd = reg_num(a);

int rs1 = reg_num(b);

int rs2 = reg_num(c);

int opcode = 0;

if (rd < 0 || rs1 < 0 || rs2 < 0)

{

printf("Line %d: invalid register in %s\n", line_no, op);

continue;

}

if      (strcmp(op, "ADD") == 0) opcode = 0x2;

else if (strcmp(op, "SUB") == 0) opcode = 0x3;

else if (strcmp(op, "AND") == 0) opcode = 0x4;

else if (strcmp(op, "OR")  == 0) opcode = 0x5;

else if (strcmp(op, "XOR") == 0) opcode = 0x6;

else if (strcmp(op, "MUL") == 0) opcode = 0x7;

else if (strcmp(op, "MAC") == 0) opcode = 0x8;

instr = (opcode << 12) | (rd << 9) | (rs1 << 6) | (rs2 << 3);

}

else if (strcmp(op, "NN") == 0)

{

int rd = reg_num(a);

if (rd < 0)

{

printf("Line %d: invalid register in NN\n", line_no);

continue;

}

instr = (0x9 << 12) | (rd << 9);

}

else if (strcmp(op, "MATMUL") == 0)

{

int rd = reg_num(a);

if (rd < 0)

{

printf("Line %d: invalid register in MATMUL\n", line_no);

continue;

}

instr = (0xA << 12) | (rd << 9);

}

else if (strcmp(op, "MOV") == 0)

{

int rd = reg_num(a);

int rs1 = reg_num(b);

if (rd < 0 || rs1 < 0)

{

printf("Line %d: invalid register in MOV\n", line_no);

continue;

}

instr = (0xB << 12) | (rd << 9) | (rs1 << 6);

}

else if (strcmp(op, "MAC2") == 0)

{

int rd = reg_num(a);

int rs1 = reg_num(b);

int rs2 = reg_num(c);

if (rd < 0 || rs1 < 0 || rs2 < 0)

{

printf("Line %d: invalid register in MAC2\n", line_no);

continue;

}

instr = (0xC << 12) | (rd << 9) | (rs1 << 6) | (rs2 << 3);

}

else

{

printf("Line %d: unknown instruction %s\n", line_no, op);

continue;

}

fprintf(fout, "%04X\n", instr);

}

fclose(fin);

fclose(fout);

printf("Assembly complete -> program.txt generated\n");

return 0;

}

CPU16.V

`timescale 1ns /1 ps

module CPU16(

input clk,

input rst,

output [15:0] dbg

);

reg [15:0] pc;

wire [15:0] instr;

instr_mem IMEM(.addr(pc), .instr(instr));

wire [3:0] opcode = instr[15:12];

wire [2:0] rd     = instr[11:9];

wire [2:0] rs1    = instr[8:6];

wire [2:0] rs2    = instr[5:3];

wire [7:0] imm8   = instr[7:0];   // used only for LOADI#imm

wire [15:0] rdata1, rdata2, r0,r1,r2,r3,r4,r5,r6,r7;

reg wen;

reg [2:0] waddr;

reg [15:0] wdata;

reg_file RF(

.clk(clk), .rst(rst), .wen(wen), .waddr(waddr), .wdata(wdata),

.raddr1(rs1), .raddr2(rs2), .rdata1(rdata1), .rdata2(rdata2),

.r0(r0),.r1(r1),.r2(r2),.r3(r3),.r4(r4),.r5(r5),.r6(r6),.r7(r7)

);

wire [15:0] alu_out;

wire [2:0] alu_op =

(opcode == 4'h2) ? 3'b000 : // ADD

(opcode == 4'h3) ? 3'b001 : // SUB

(opcode == 4'h4) ? 3'b010 : // AND

(opcode == 4'h5) ? 3'b011 : // OR

(opcode == 4'h6) ? 3'b100 : // XOR

(opcode == 4'h7) ? 3'b101 : // MUL

(opcode == 4'h8) ? 3'b110 : // MAC

3'b111;                      // reserved

ALU16 ALU(.a(rdata1), .b(rdata2), .acc(r1), .op(alu_op), .result(alu_out));

wire [31:0] nn_out;

NN NN0(.x1(rdata1), .x2(rdata2), .w1(rdata1), .w2(rdata2), .b(r7), .y(nn_out));

wire [15:0] c00,c01,c10,c11;

Mat_mul MM(

.a00(r0), .a01(r1), .a10(r2), .a11(r3),

.b00(r4), .b01(r5), .b10(r6), .b11(r7),

.c00(c00), .c01(c01), .c10(c10), .c11(c11)

);

wire [15:0] mac_out;

MAC MAC0(

.a(rdata1),

.b(rdata2),

.acc(r1),

.result(mac_out)

);

assign dbg = r1;

always @(posedge clk or posedge rst) begin

if (rst) begin

pc    <= 0;

wen   <= 0;

waddr <= 0;

wdata <= 0;

end else begin

wen <= 0;

pc  <= pc + 2;

case(opcode)

4'h0: ; // Eat 5*

4'h1: begin // LoadI

wen   <= 1;

waddr <= rd;

wdata <= {8'b0, imm8};

end

4'h2: begin // ADD

wen   <= 1;

waddr <= rd;

wdata <= alu_out;

end

4'h3: begin // SUB

wen   <= 1;

waddr <= rd;

wdata <= alu_out;

end

4'h4: begin // AND

wen   <= 1;

waddr <= rd;

wdata <= alu_out;

end

4'h5: begin // OR

wen   <= 1;

waddr <= rd;

wdata <= alu_out;

end

4'h6: begin // XOR

wen   <= 1;

waddr <= rd;

wdata <= alu_out;

end

4'h7: begin // MUL

wen   <= 1;

waddr <= rd;

wdata <= alu_out;

end

4'h8: begin // MAC (acc = R1)

wen   <= 1;

waddr <= rd;

wdata <= alu_out;

end

4'h9: begin // NN

wen   <= 1;

waddr <= rd;

wdata <= nn_out[15:0];

end

4'hA: begin // MMAT00

wen   <= 1;

waddr <= rd;

wdata <= c00;

end

4'hB: begin // MOV R,R

wen   <= 1;

waddr <= rd;

wdata <= rdata1;

end

//            MOV rd rs1

//            4'hC: begin

//                wen   <= 1;

//                waddr <= rd;

//                wdata <= rdata1;   // R[rd] ? R[rs1]

//            end

4'hC: begin

wen   <= 1;

waddr <= rd;

wdata <= mac_out;   // R[rd] ? R[rs1]

end

4'hD: begin

wen   <= 1;

waddr <= rd;

wdata <= alu_out;

end

4'hF: begin // EXIT

$display("FINAL: R0=%h R1=%h R2=%h", r0,r1,r2);

#5 $finish;

end

default: ; // NOP

endcase

end

end

endmodule

instr_mem.v

`timescale 1ns / 1ps

//////////////////////////////////////////////////////////////////////////////////

// Company:

// Engineer:

//

// Create Date: 26.04.2026 16:43:42

// Design Name:

// Module Name: instr_mem

// Project Name:

// Target Devices:

// Tool Versions:

// Description:

//

// Dependencies:

//

// Revision:

// Revision 0.01 - File Created

// Additional Comments:

//

//////////////////////////////////////////////////////////////////////////////////

module instr_mem(

input [15:0] addr,

output [15:0] instr

);

reg [15:0] mem [0:255];

// Load machine code from program.txt

/* initial begin

$readmemh(

"E:/vhdl programs/CPU16modular/CPU16modular.srcs/sim_1/new/program.txt",

mem

);

*/

integer i;

initial begin

for (i = 0; i < 256; i = i + 1)

mem[i] = 16'h0000; // default NOP

$readmemh("E:/vhdl programs/CPU16modular/CPU16modular.srcs/sim_1/new/program.txt", mem);

$display("Instruction Loaded:");

$display("mem[0] = %h", mem[0]);

$display("mem[1] = %h", mem[1]);

$display("mem[2] = %h", mem[2]);

end

// Since PC increases by 2:

// PC = 0,2,4,6...

// convert to memory index using >> 1

assign instr = mem[addr >> 1];

endmodule

reg_file.v

module reg_file(

input clk,

input rst,

input wen,

input [2:0] waddr,

input [15:0] wdata,

input [2:0] raddr1,

input [2:0] raddr2,

output reg [15:0] rdata1,

output reg [15:0] rdata2,

output [15:0] r0,r1,r2,r3,r4,r5,r6,r7  // ? Move outputs here

);

reg [15:0] regs [7:0];

integer i;

always @(posedge clk or posedge rst) begin

if (rst) begin

for (i=0; i<8; i=i+1)

regs[i] <= 16'h0;

end else if (wen) begin

regs[waddr] <= wdata;

end

end

always @(*) begin

rdata1 = regs[raddr1];

rdata2 = regs[raddr2];

end

// ? Debug outputs

assign r0 = regs[0];

assign r1 = regs[1];

assign r2 = regs[2];

assign r3 = regs[3];

assign r4 = regs[4];

assign r5 = regs[5];

assign r6 = regs[6];

assign r7 = regs[7];

endmodule

ALU16.v

`timescale 1ns / 1ps

module ALU16(

input [15:0] a,      // rs1

input [15:0] b,      // rs2

input [15:0] acc,    // R1 (accumulator)

input [2:0] op,

output reg [15:0] result

);

always @(*) begin

case(op)

3'b000: result = a + b;                    // ADD

3'b001: result = a - b;                    // SUB

3'b010: result = a & b;                    // AND

3'b011: result = a | b;                    // OR

3'b100: result = a ^ b;                    // XOR

3'b101: result = a * b;                    // MUL

//Not of Acc till now just a

3'b110: result = ~a ;

default: result = 16'b0;

endcase

end

endmodule

NN.v(neuron)

`timescale 1ns / 1ps

module NN(

input [15:0] x1,

input [15:0] x2,

input [15:0] w1,

input [15:0] w2,

input [15:0] b,

output [31:0] y

);

assign y = (x1 * w1) + (x2 * w2) + b;

endmodule

Mat_mul.v

module Mat_mul(

input [15:0] a00,a01,a10,a11,

input [15:0] b00,b01,b10,b11,

output [15:0] c00,c01,c10,c11

);

assign c00 = (a00*b00) + (a01*b10);

assign c01 = (a00*b01) + (a01*b11);

assign c10 = (a10*b00) + (a11*b10);

assign c11 = (a10*b01) + (a11*b11);

endmodule

MAC.v

`timescale 1ns / 1ps

module MAC(

input [15:0] a,

input [15:0] b,

input [15:0] acc,

output [31:0] result

);

assign result = acc + (a * b);

endmodule

Test Bench code:

Cpu16_tb_modular.v

`timescale 1ns / 1ps

module CPU16_tb_moduler;

// Inputs

reg clk;

reg rst;

// Output

wire [15:0] dbg;

// Device Under Test

CPU16 uut (

.clk(clk),

.rst(rst),

.dbg(dbg)

);

// Clock Generation

// 10ns clock period

initial begin

clk = 0;

forever #5 clk = ~clk;

end

// Test Sequence

initial begin

clk = 0;

// Apply Reset

rst = 1;

#20;

rst = 0;

// Run simulation long enough

#1000;

$finish;

end

// Monitor Values

initial begin

$monitor(

"Time=%0t | rst=%b | dbg(R1)=%h", $time, rst, dbg);

end

endmodule

Console Output after running Simulation:

run 1000ns

Instruction Loaded:

mem[0] = 1001

mem[1] = 1202

mem[2] = 1403

Time=0 | rst=1 | dbg(R1)=0000

Time=20000 | rst=0 | dbg(R1)=0000

Time=45000 | rst=0 | dbg(R1)=0002

Time=125000 | rst=0 | dbg(R1)=000c

FINAL: R0=0069 R1=000c R2=fffc

$finish called at time : 180 ns : File "E:/vhdl programs/CPU16modular/CPU16modular.srcs/sources_1/new/CPU16.v" Line 160

INFO: [USF-XSim-96] XSim completed. Design snapshot 'CPU16_tb_moduler_behav' loaded.

INFO: [USF-XSim-97] XSim simulation ran for 1000ns

launch_simulation: Time (s): cpu = 00:00:03 ; elapsed = 00:00:07 . Memory (MB): peak = 1879.258 ; gain = 0.414

Waveform:

bo